This folder contains the model files for the Hestadbäcken and Tullstorpån catchment models respectively used in the linked manuscript. Each catchment folder is divided in two subfolders: 1) model calibration and validation, 2) climate forecasts and scenario modelling.

In order to run the scenario or climate forecasts, copy the specific files and rename them to the following:

- CropData

- filedir

- GeoClass

- Geodata

- info

- par

- Pobs

- Tobs

Double click on HYPE_5_18_0 to run the model, for the model to provide you with output files you need a folder named results.

If you only want to test the impact of climate change, choose the current scenario with a specific climate model and RCP. For example, the KNMI2.6 choose: CropData_current, GeoClass_current, Geodata_current, par_current. You can run the model for different timeperiods and compare the outcomes by separately running info_2030 (2022-2035), info_2060 (2050-2064), info_2100 (2085-2099).

You can combine different scenarios and climate trajectories. If you only want to test one scenario, choose the current version for the other files. For example, if you want to test the stream mitigation scenario under the KNMI model RCP2.6 for the period 2022-2035, choose the following files: CropDate_Current, GeoClass_Current, Geodata_StreamMitigation, info_2030, par_StreamMitigation, Pobs_KNMI2.6, Tobs_KNMI2.6.

Finally, you can combine multiple management practices. For example, to combine Stream Mitigation, 20% reduction in fertiliser, and cover crops, choose: CropDate_20%Fert, GeoData_StreamMitigation, par_StreamMitigation. You can select the info file based on the time period and select the Pobs and Tobs file based on the climate model and RCP.

The outputs are presented as text files in the results folder based on the basin number. In this case, the upstream outputs are named 000002, the midstream 000003, and the downstream 000004. You can add or remove outputs by amending the info file. For the meaning of the outputs, I refer to the HYPE_file_reference.