Change detection of mountain birch using multi-temporal ALS point clouds

MATTIAS NYSTRÖM*†, JOHAN HOLMGREN† and HÅKAN OLSSON†

† Swedish University of Agricultural Sciences, Department of Forest Resource Management,
SE-90183 Umeå, Sweden

Use of multi-temporal laser scanner data is potentially a very efficient method for monitoring of vegetation changes, for example at the alpine tree line. In this study, methods for relative calibration of multi-temporal ALS data sets and detection of experimental changes of tree cover in the forest-tundra ecotone was tested in northern Sweden (68° 20' N, 19° 01' E). Trees were either partly or totally removed on six meter radius sample plots to simulate two classes of biomass change. Histogram matching was successfully used to calibrate the laser metrics from the two data sets and sample plots were then classified into three change classes. The proportion of vegetation returns from the canopy was the most important explanatory variable which provided an overall accuracy of 88%. The classification accuracy was clearly dependent on the density of the forest.

# 1. Introduction

The effect of ongoing climate change on sub-arctic and alpine forests has led to increased interest in monitoring potential changes in the forest-tundra ecotone. In addition to climate change, insect damage, browsing pressure by herbivores as well as anthropogenic impacts will contribute to changes in the sub-arctic forest-tundra ecotone. These changes are difficult to monitor with manual methods because of the complex mosaic pattern of the ecotone.

Airborne laser scanning (ALS) can efficiently be used to estimate tree height, biomass, and canopy closure in the forest-tundra ecotone (Nyström et al. 2012). In the future, series of ALS data will become available collected with various resolutions, scanning systems, system parameters, etc. Therefore, research is needed to find methods for efficient calibration and change detection of multi-temporal data.

However, so far there is limited research on using multi-temporal ALS data to detect areas with removed trees or estimation of forest growth. Hyyppä et al. (2003) and Yu et al. (2008, 2004, 2006) used high resolution ALS data on a single tree level to estimate forest growth and detect harvested trees. St-Onge et al. (2004) used ALS data acquired five years in between to detect fallen trees, individual tree growth and overall growth. Næsset and Gobakken (2005) estimated forest growth in a boreal forest using ALS data with approximately 1 m-2 acquired two years in between. Corresponding metrics from the two ALS datasets were compared. Mean tree height, basal area and volume were regressed against the laser metrics and a significant forest growth in all three variables was predicted. Solberg et al. (2006) mapped changes in leaf area index (LAI) due to insect attacks by calibration of ALS data to field measurements. Hopkinson et al. (2008) found that higher precision for growth estimation could be achieved with longer time intervals (five years compared to one year). Næsset (2009) conclude that different ALS sensors, flying altitudes and pulse repetition frequencies affect height-related as well as canopy density-related metrics. Vastaranta et al. (2011) detected snow-damaged trees at a test site in southern Finland using the difference in ALS measured canopy heights.

Relative calibration methods have earlier been developed for analysis of optical satellite image data (Olsson 1993; Coppin et al. 2004). It is of interest to evaluate if these calibration methods are suitable also for analysis of ALS data.

In this study, we used two ALS acquisitions from two time points with different scanning systems, system parameters, point densities, and flying altitudes. Between the two acquisitions, sample plots were placed out and a proportion of the trees were cut to simulate afforestation. The objectives were to (1) validate the effect of a histogram matching algorithm when comparing the two data sets, and to (2) identify metrics from ALS data that are efficient to detect changes of vegetation in the sub-alpine tree line ecotone using supervised classification.

# 2. Material and methods

## 2.1 Study area

The study area is about two km2 and located six km southeast of Abisko in northern Sweden, centred on Lat. N 68°20’, Long. E 19°01’ (figure 1(a)). The dominating specie is mountain birch (Betula pubescens ssp. czerepanovii), but some Junipers (Juniperus communis), Rowans (Sorbus aucuparia), and Willows (Salix spp.) taller than 1.5 m are also present. The birches in the study area are of the multi-stem type with several stems often sharing the same root system (polymorphism). Sample plots were placed in the ecotone between birch forest and tundra (denoted forest-tundra ecotone). This ecotone was characterized by a mosaic pattern of forest and alpine heath vegetation, with elevations from 410 – 670 m a.s.l.

[Figure 1]

## 2.2 Laser data acquisitions

Laser data were acquired under leaf-on conditions at two occasions with different laser scanners (table 1). The first scanning was done in 2008 with a TopEye MkII (denoted TopEye) mounted on a helicopter and the second scanning was done in 2010 with an Optech ALTM Gemini (denoted Optech), mounted within a fixed-wing aircraft. In both cases, scanning was performed with the flying direction orthogonal to the main slope.

[Table 1]

## 2.3 Field data

The field inventory was carried out between the two laser data acquisitions, during two weeks in June 2010 (43 sample plots), see table 2 for details. In addition, 53 sample plots, inventoried in August 2009 and June 2010, from a previous study in the area (Nyström et al. 2012) were used. These 53 sample plots had ten meters radius, taller trees and were in average located in areas with older forest compared to the 43 sample plots inventoried in this study. Only six meters radius of the sample plots from the previous study were used in the analysis and the sample plots were revisited in 2010 to assure they were unchanged. The 43 new sample plots had six meters radius and were subjectively selected to match the following criteria: (1) 90% of the trees in the range 1.5-2.5 m tall, (2) no trees taller than 3.5 m, and (3) no dominating bush layer (0-1.5 m) on the sample plot. The centre position of all sample plots was measured with sub-dm accuracy. Figure 1(b) show one of the sample plots from the field data collection in 2010.

Each sample plot was assigned one of the following changes: (1) reference, no removal of trees, (2) removal of 50% of the trees taller than 1.5 m, and (3) removal of 100% of the trees taller than 1.5 m. Spatial location and height distribution of removed trees were as evenly distributed as possible on each sample plot. The treatment was extended to seven meters radius to minimize problems with horizontal dislocation between field and laser data, but still only six meters radius was used when extracting laser data.

To evaluate the classification, sample plots were divided into three density categories depending on the total number of trees taller than 1.5 m: (1) low density, 5-10 trees, (2) medium density, 11-50 trees, and (3) high density, 51-100 trees. Table 2 gives a summary of the tree densities and the change classes.

[Table 2]

## 2.4 Laser data processing

TerraScan (2010) was used to classify the point cloud into ground and non-ground in the same way as Nyström et al. (2012). The statistical software R (R Development Core Team, 2010) and in house developed programs were used to further process the ALS point cloud. No removal of overlapping data was done in either data set.

A digital elevation model (DEM) with 0.5 m raster cell size representing the ground level was created for each data set in the following way. First, the mean elevation of ground classified points was calculated for each cell. Secondly, empty cells were assigned elevation values by TIN interpolating the filled cells. Canopy heights (CH) were calculated by subtracting the DEM from the z-value of each laser return. A digital surface model (DSM) with 0.5 m raster cell size was created by assigning each grid cell the maximum z-value for laser returns classified as non-ground. A normalized DSM (nDSM) was calculated by subtracting the DEM from the DSM. All data points above 15 m were omitted in CH and nDSM to avoid false reflections (the 15 m limit was chosen given that no trees taller than 10.5 m were found in the area).

A height threshold (Nilsson 1996) of 0.7 m was used when calculating metrics from the ALS point cloud above ground (CH and nDSM). The threshold was chosen to obtain reliable laser metrics for sample plots with trees around 1.5 m tall.

All the following laser metrics were created from both nDSM and CH. Height percentiles () in steps of 20 were calculated using canopy heights above the height threshold. In addition, height percentiles were also calculated for  and  since these are strong indicators of the height of vegetation. Ten vertical canopy density metrics () were estimated in accordance to e.g. Næsset and Gobakken (2008) using the height threshold as the lower limit and  as the upper limit. A sum of squared canopy heights () was calculated by taking the sum of squared canopy heights divided by the number of canopy height measurements above the height threshold (Nyström et al. 2012). Vegetation ratio () was calculated in two ways: (1) by dividing the number of laser returns above the height threshold with the total number of returns inside the sample plot and (2) using point-weighted approach (denoted ) according to Nyström et al. (2012) using radius 0.59 m for TopEye and 1.00 m for Optech. The returns used to calculate the vegetation ratio was either all returns (denoted ) or only first returns (denoted ).

## 2.5 Histogram matching

An area of 1.8 km2 (figure 1(a)) was used to calculate laser metrics for 10x10 m pixels. Histogram matching was used to calibrate the metrics from the two laser data acquisitions to a common distribution. Cumulative histograms were created for each laser metric in the two data sets using 100 bins with minimum and maximum values from each laser metric as bounds. In the histogram matching process, the values were interpolated between the bins using piecewise cubic hermite interpolation (Fritsch and Carlson 1980). The histograms from the Optech laser metrics were matched to the histograms from the TopEye laser metrics. In total 18450 pixels were inside the bounding box (figure 1(a)), but several pixels had no laser metrics because no vegetation was present. Therefore only 10593 pixels were used to create the histograms.

To simulate afforestation, the first laser data collection (2008) was used as “after” and the second laser data collection (2010) as “before”. The field reference plots were used to evaluate the similarity of the laser metrics after histogram matching was conducted. The reference plots had normal growth and no unusual changes according to the field survey. The measures used were relative  (), and relative  
() calculated from the unchanged reference sample plots ():

where  is laser metric  of sample plot  from the TopEye data and  is the same laser metric () from the same sample plot () in the Optech data,  is the number of observations, and  is the mean value of laser metric , i.e. mean value of  and . This provided an opportunity to check both the success of the histogram matching, and to obtain an indication of which laser metrics that provided most similar data for the normally developed vegetation.

## 2.6 Classification

The sample plots were classified into the three change classes using Linear Discriminant Analysis (LDA) in order to check which laser metrics that best discriminated changed from unchanged vegetation. The MASS-package (Venables, 2002) in the statistical software R (R Development Core Team, 2010) was used for the classification. Prior probabilities were set to 1/3 for each of the three classes. Relative difference of the laser metrics was used as explanatory variables:

A small number () was added in the denominator to avoid division by zero. The classification was based on either one or a combination of two explanatory variables 
(). Leave-one-out cross-validation was used to calculate classification accuracy.

# 3. Results

Each laser metric from the second scanning (Optech) was histogram matched to the histogram from the first scanning (TopEye). The cumulative histograms created for the 95th height percentile () and the vegetation ratio from nDSM () is shown in figure 2. Figure 3 shows one-to-one plots of the 95th height percentile and the vegetation ratio without calibration and calibrated using histogram matching. In the figure it can be seen that after histogram matching was applied, the relationship between the metrics from the different sensors is no longer curved.

[Figure 2] [Figure 3]

The similarity of laser metrics between the two ALS acquisitions was evaluated by calculating  and  for the reference sample plots with and without histogram matching applied. In table 3 it can be seen that laser metrics calculated from nDSM tend to have lower  and  than metrics calculated using CH.  has always a low value when histogram matching was applied.

[Table 3]

Table 4 shows overall cross-validated classification accuracy using only one laser metric as explanatory variable. It can be seen that laser metrics based on the density of the vegetation tend to have higher overall classification accuracy. There was only 1 % higher classification accuracy when the best combination of two metrics ( and ) was used. Table 5 shows the error matrices for a height percentile (), a density metric () and a combination of a height percentile and a density metric 
( and ). Almost none of the 50% changed sample plots were correctly classified using only the height percentile. Considerably higher classification accuracy for the 50% changed sample plots was achieved using only a measure of density.

[Table 4] [Table 5]

# 4. Discussion

This is to our knowledge the first study that reports the performance of histogram matching for multi-temporal relative calibration of ALS data. The cumulative histograms (figure 2(b)) created for the vegetation ratio () were found to be almost identical after the histogram matching was applied. Consequently, the calibrated data became also better aligned to the one-to-one line (figure 3(c-d)).

and  (table 3) were calculated using the 68 reference sample plots without experimental changes to evaluate similarity between the two data sets. The height percentiles had considerably lower  than the density metrics. Bater et al. (2011) compared four acquisitions from the same day and also noted that the height percentiles were more stable than density metrics.  has always a low value when histogram matching is used. When calculating  using only the sample plots from the taller forest (previously inventoried sample plots),  has a value very close to zero for all metrics. This indicate that some of the change in the reference plots with lower forest might be due to forest growth, but can also be due to difficulties to measure low forest with ALS.

The laser metrics created using nDSM values have lower  for the reference plots and higher classification accuracy than metrics created from CH. Multi-temporal data requires measures to be spatially normalized at each time point to avoid problems with uneven distribution of laser points. There is a need for further research on spatial normalization methods when analysing multi-temporal data.

The reason for the low classification accuracy using only a height percentile can be that trees taller than 1.5 m still remained on the sample plots. The highest classification accuracy was achieved using a density metric (). Divided into the three density classes, classification accuracy was 71%, 81% and 95% in low-, medium- and high density forest, respectively. Hence, classification accuracy was clearly dependent on the forest density.

The footprint was about 0.5 m with both laser scanners regardless of the difference in flying altitude. In the case of the Optech data, the ground is however not fully illuminated, which means that some vegetation might be omitted. How this affected the results in this study is ambiguous, but when comparing the 3D point clouds from the two acquisitions it is evident that the denser TopEye data clearly show the spatial pattern of the vegetation much better than the more sparse Optech data.

High classification accuracy (88 %) was achieved using a measure of vegetation density derived from data acquired at two time points with different sensors, flying altitude and scanning pattern. This was obtained by using metrics corrected for uneven distribution of laser points. Analysis of histograms is important to verify the similarity between ALS data from two time points. Histogram matching reduced the difference between the two data sets for most metrics and is likely to be a straight forward way to produce change imagery from multi-temporal ALS data sets.

# 5. Acknowledgements

This study is part of the research programme Environmental Mapping and Monitoring with Airborne laser and digital images (EMMA), financed by the Swedish Environmental Protection Agency. The laser data sets were acquired through cooperation with the University of Lund and the new national elevation model project (NNH) at the Swedish National Land Survey. The Abisko Scientific Research Station is acknowledged for supporting the work with field data acquisition.

# References

Bater, C.W., Wulder, M.A., Coops, N.C., Nelson, R.F., Hilker, T. and Næsset, E., 2011, Stability of Sample-Based Scanning-LiDAR-Derived Vegetation Metrics for Forest Monitoring. Geoscience and Remote Sensing, IEEE Transactions on, 49, pp. 2385-2392.

Coppin, P., Jonckheere, I., Nackaerts, K., Muys, B. and Lambin, E., 2004, Digital change detection methods in ecosystem monitoring: a review. International Journal of Remote Sensing, 25, pp. 1565-1596.

Fritsch, F.N. and Carlson, R.E., 1980, Monotone Piecewise Cubic Interpolation. Siam Journal on Numerical Analysis, 17, pp. 238-246.

Hopkinson, C., Chasmer, L. and Hall, R.J., 2008, The uncertainty in conifer plantation growth prediction from multi-temporal lidar datasets. Remote Sensing of Environment, 112, pp. 1168-1180.

Hyyppä, J., Xiaowei, Y., Rönnholm, P., Kaartinen, H. and H, H., 2003, Factors Affecting Object-Oriented Forest Growth Estimates Obtained Using Laser Scanning. The photogrammetric journal of Finland, 18, pp. 16-31.

Næsset, E., 2009, Effects of different sensors, flying altitudes, and pulse repetition frequencies on forest canopy metrics and biophysical stand properties derived from small-footprint airborne laser data. Remote Sensing of Environment, 113, pp. 148-159.

Næsset, E. and Gobakken, T., 2005, Estimating forest growth using canopy metrics derived from airborne laser scanner data. Remote Sensing of Environment, 96, pp. 453-465.

Næsset, E. and Gobakken, T., 2008, Estimation of above- and below-ground biomass across regions of the boreal forest zone using airborne laser. Remote Sensing of Environment, 112, pp. 3079-3090.

Nilsson, M., 1996, Estimation of tree weights and stand volume using an airborne lidar system. Remote Sensing of Environment, 56, pp. 1-7.

Nyström, M., Holmgren, J. and Olsson, H., 2012, Prediction of tree biomass in the forest–tundra ecotone using airborne laser scanning. Remote Sensing of Environment, 123, pp. 271-279.

Olsson, H., 1993. Regression functions for multitemporal relative calibration of Thematic Mapper data over boreal forests. Remote Sensing of Environment, 46, pp. 89-102.

R Development Core Team, 2011. R: A language and environment for statistical computing. R Foundation for Statistical Computing, Vienna, Austria. ISBN 3-900051-07-0, URL http://www.R-project.org/.

Soininen, A., 2010, TerraScan User's Guide. TerraSolid Ltd.

Solberg, S., Næsset, E., Hanssen, K.H. and Christiansen, E., 2006, Mapping defoliation during a severe insect attack on Scots pine using airborne laser scanning. Remote Sensing of Environment, 102, pp. 364-376.

St-Onge, B. and Vepakomma, U., 2004, Assessing Forest Gap Dynamics and Growth Using Multi-Temporal Laser-Scanner Data. In International Archieves of Photogrammetry, Remote Sensing and Spatial Information Sciences XXXVI 8/W2, 3-6 October, 2004, Freiburg, Germany, pp. 173-178.

Vastaranta, M., Korpela, I., Uotila, A., Hovi, A. and Holopainen, M., 2011, Mapping of snow-damaged trees based on bitemporal airborne LiDAR data. European Journal of Forest Research, pp. 1-12.

Venables, W. N. and Ripley, B. D., 2002. Modern Applied Statistics with S. Fourth Edition. Springer, New York. ISBN 0-387-95457-0.

Yu, X., Hyyppä, J., Kaartinen, H., Maltamo, M. and Hyyppä, H., 2008, Obtaining plotwise mean height and volume growth in boreal forests using multi-temporal laser surveys and various change detection techniques. International Journal of Remote Sensing, 29, pp. 1367-1386.

Yu, X.W., Hyyppä, J., Kaartinen, H. and Maltamo, M., 2004, Automatic detection of harvested trees and determination of forest growth using airborne laser scanning. Remote Sensing of Environment, 90, pp. 451-462.

Yu, X.W., Hyyppä, J., Kukko, A., Maltamo, M. and Kaartinen, H., 2006, Change detection techniques for canopy height growth measurements using airborne laser scanner data. Photogrammetric Engineering and Remote Sensing, 72, pp. 1339-1348.

Figure 1. (a) The study area in northern Sweden. The red dots in the enlarged map are the sample plots and the black polygon is the area used to create cumulative histograms for histogram matching. © Lantmäteriet, I 2010/0345. (b) One of the sample plots from the field data collection. This sample plot was classified to have medium tree density. The red/yellow measuring pole is 3 m in length.

Figure 2. The red dashed line is the cumulative histogram of Optech after histogram matching was applied to match the histogram of TopEye (black line). (a) 95th height percentile (). (b) Vegetation ratio ().

Figure 3. One-to-one plot before and after histogram matching was applied. The red line is the one-to-one line. Only every fifth point is plotted. (a) 95th height percentile (). (b) Vegetation ratio ().

Table 1. Summary of the laser scanner properties and flight parameters of the two laser data acquisitions.

| Parameter | TopEye MkII | Optech ALTM Gemini |
| --- | --- | --- |
| Scanning date | Aug. 1, 2008 | Aug. 20, 2010 |
| Flight altitude (above ground) | 500 m | 1740 m |
| Footprint | 0.5 m | 0.5 m |
| Pulse repetition frequency | 50 kHz | 70 kHz |
| Scan frequency | 35 Hz | 37 Hz |
| Wave length | 1064 nm | 1064 nm |
| Pulse length | 4 ns (1.2 m) | 6.8 ns (2.0 m) |
| Scan type | Palmer | Oscillating mirror |
| Scan width (across flight dir.) | ±20° | ±20° |
| Scan width (along flight dir.) | ±14° | 0° |
| Point extraction | Up to 2 per pulse | Up to 4 per pulse |
| Minimum point density † | 3.0 m-2 | 0.7 m-2 |
| Average point density † | 12.0 m-2 | 1.2 m-2 |
| Maximum point density † | 34.8 m-2 | 2.4 m-2 |

† based on the 96 sample plots used in this article.

Table 2: Summary of the field data.

|  | Change class (Tree density class: low/medium/high) |  |  |
| --- | --- | --- | --- |
| Collected | Reference | 50% | 100% |
| Previous study † | 53 (6/17/30) | 0 | 0 |
| This study | 15 (5/5/5) | 13 (5/4/4) | 15 (6/5/4) |

† Nyström et al. (2012)

Table 3.  and  for the 68 reference sample plots. In each of the following categories , , , and , only the metric with lowest  after histogram matching is presented.

|  | (%) |  |  | (%) |  |
| --- | --- | --- | --- | --- | --- |
| Laser metric | Hist. match | Not cal.† |  | Hist. match | Not cal.† |
|  | 6.0 | 9.2 |  | -0.4 | 6.8 |
|  | 6.7 | 8.5 |  | -0.2 | 4.5 |
|  | 11.5 | 14.5 |  | -1.2 | 6.8 |
|  | 13.7 | 13.6 |  | -1.6 | -0.6 |
|  | 14.5 | 15.5 |  | -0.9 | 2.9 |
|  | 15.2 | 17.6 |  | -1.9 | 5.3 |
|  | 16.4 | 18.1 |  | -1.8 | -1.2 |
|  | 16.5 | 23.4 |  | -2.4 | 12.0 |
|  | 19.4 | 22.4 |  | -2.3 | 5.0 |
|  | 19.4 | 22.4 |  | -2.3 | 5.0 |
|  | 21.2 | 32.8 |  | -2.8 | 18.3 |
| Mean: | 14.6 | 18.0 |  | 1.6‡ | 6.2‡ |

† Not calibrated.

‡ Mean value calculated using absolute values.

Table 4. Overall classification accuracy after leave-one-out cross-validation in the three change classes using one explanatory variable. In each of the following categories , , , and , only the metric with highest overall classification accuracy is presented.

|  | Classification acc. (%) |  |
| --- | --- | --- |
| Laser metric | Hist. match | Not cal.† |
|  | 88 | 86 |
|  | 86 | 86 |
|  | 85 | 82 |
|  | 84 | 80 |
|  | 83 | 75 |
|  | 82 | 79 |
|  | 82 | 79 |
|  | 81 | 80 |
|  | 80 | 84 |
|  | 69 | 73 |
|  | 67 | 72 |

† Not calibrated.

Table 5. Error matrix for leave-one-out cross-validated supervised classification using histogram matched metrics: (1) , (2)  and (3)  and .

|  |  | True |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | and |  |  |
|  |  | Ref. | 50% | 100% | Ref. | 50% | 100% | Ref. | 50% | 100% |
| Predicted | Ref. | 65 | 8 | 3 | 64 | 3 | 0 | 66 | 3 | 0 |
|  | 50 % | 3 | 3 | 2 | 3 | 8 | 3 | 1 | 9 | 5 |
|  | 100% | 0 | 2 | 10 | 1 | 2 | 12 | 1 | 1 | 10 |

* Corresponding author. E-mail: mattias.nystrom@slu.se